# How to control a HD44780-based Character-LCD

(Industry-Standard-Character-LCD)
General info and code-examples

### 1.1. Pin assignment

The pin assignment shown in Table 1.1. is the industry standard for character LCD-modules with a maximum of 80 characters.
The pin assignment shown in Table 1.2. is the industry standard for character LCD-modules with more than 80 characters.

To be sure always check the manufacturers datasheet!
To locate pin 1 on a module check the manufacturers datasheet!

| Table1.1., Pin assignment for <= 80 character displays |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin number | Symbol | Level | I/O | Function |
| 1 | Vss | - | - | Power supply (GND) |
| 2 | Vcc | - | - | Power supply (+5V) |
| 3 | Vee | - | - | Contrast adjust |
| 4 | RS | 0/1 | I | 0 = Instruction input 1 = Data input |
| 5 | R/W | 0/1 | I | 0 = Write to LCD module 1 = Read from LCD module |
| 6 | E | 1, 1-->0 | I | Enable signal |
| 7 | DB0 | 0/1 | I/O | Data bus line 0 (LSB) |
| 8 | DB1 | 0/1 | I/O | Data bus line 1 |
| 9 | DB2 | 0/1 | I/O | Data bus line 2 |
| 10 | DB3 | 0/1 | I/O | Data bus line 3 |
| 11 | DB4 | 0/1 | I/O | Data bus line 4 |
| 12 | DB5 | 0/1 | I/O | Data bus line 5 |
| 13 | DB6 | 0/1 | I/O | Data bus line 6 |
| 14 | DB7 | 0/1 | I/O | Data bus line 7 (MSB) |

| Table 1.2., Pin assignment for > 80 character displays |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin number | Symbol | Level | I/O | Function |
| 1 | DB7 | 0/1 | I/O | Data bus line 7 (MSB) |
| 2 | DB6 | 0/1 | I/O | Data bus line 6 |
| 3 | DB5 | 0/1 | I/O | Data bus line 5 |
| 4 | DB4 | 0/1 | I/O | Data bus line 4 |
| 5 | DB3 | 0/1 | I/O | Data bus line 3 |
| 6 | DB2 | 0/1 | I/O | Data bus line 2 |
| 7 | DB1 | 0/1 | I/O | Data bus line 1 |
| 8 | DB0 | 0/1 | I/O | Data bus line 0 (LSB) |
| 9 | E1 | 1, 1->0 | I | Enable signal row 0 & 1 |
| 10 | R/W | 0/1 | I | 0 = Write to LCD module 1 = Read from LCD module |
| 11 | RS | 0/1 | I | 0 = Instruction input 1 = Data input |
| 12 | Vee | - | - | Contrast adjust |
| 13 | Vss | - | - | Power supply (GND) |
| 14 | Vcc | - | - | Power supply (+5V) |
| 15 | E2 | 1, 1->0 | I | Enable signal row 2 & 3 |
| 16 | n.c. |  |  |  |

### 1.2. Instruction set

| Table 1.3. HD44780 instruction set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instruction | Code |  |  |  |  |  |  |  |  |  | Description | Execution time** |
|  | RS | R/W | DB7 | DB6 | DB5 | DB4 | DB3 | DB2 | DB1 | DB0 |  |  |
| Clear display | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | Clears display and returns cursor to the home position (address 0). | 1.64mS |
| Cursor home | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | * | Returns cursor to home position (address 0). Also returns display being shifted to the original position. DDRAM contents remains unchanged. | 1.64mS |
| Entry mode set | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | I/D | S | Sets cursor move direction (I/D), specifies to shift the display (S). These operations are performed during data read/write. | 40uS |
| Display On/Off control | 0 | 0 | 0 | 0 | 0 | 0 | 1 | D | C | B | Sets On/Off of all display (D), cursor On/Off (C) and blink of cursor position character (B). | 40uS |
| Cursor/display shift | 0 | 0 | 0 | 0 | 0 | 1 | S/C | R/L | * | * | Sets cursor-move or display-shift (S/C), shift direction (R/L). DDRAM contents remains unchanged. | 40uS |
| Function set | 0 | 0 | 0 | 0 | 1 | DL | N | F | * | * | Sets interface data length (DL), number of display line (N) and character font(F). | 40uS |
| Set CGRAM address | 0 | 0 | 0 | 1 | CGRAM address |  |  |  |  |  | Sets the CGRAM address. CGRAM data is sent and received after this setting. | 40uS |
| Set DDRAM address | 0 | 0 | 1 | DDRAM address |  |  |  |  |  |  | Sets the DDRAM address. DDRAM data is sent and received after this setting. | 40uS |
| Read busy-flag and address counter | 0 | 1 | BF | CGRAM / DDRAM address |  |  |  |  |  |  | Reads Busy-flag (BF) indicating internal operation is being performed and reads CGRAM or DDRAM address counter contents (depending on previous instruction). | 0uS |
| Write to CGRAM or DDRAM | 1 | 0 | write data |  |  |  |  |  |  |  | Writes data to CGRAM or DDRAM. | 40uS |
| Read from CGRAM or DDRAM | 1 | 1 | read data |  |  |  |  |  |  |  | Reads data from CGRAM or DDRAM. | 40uS |

Remarks:
- DDRAM = Display Data RAM.
- CGRAM = Character Generator RAM.
- DDRAM address corresponds to cursor position.
- * = Don't care.
- ** = Based on Fosc = 250KHz.

| Table1.4. Bit names |  |  |
| --- | --- | --- |
| Bit name | Settings |  |
| I/D | 0 = Decrement cursor position | 1 = Increment cursor position |
| S | 0 = No display shift | 1 = Display shift |
| D | 0 = Display off | 1 = Display on |
| C | 0 = Cursor off | 1 = Cursor on |
| B | 0 = Cursor blink off | 1 = Cursor blink on |
| S/C | 0 = Move cursor | 1 = Shift display |
| R/L | 0 = Shift left | 1 = Shift right |
| DL | 0 = 4-bit interface | 1 = 8-bit interface |
| N | 0 = 1/8 or 1/11 Duty (1 line) | 1 = 1/16 Duty (2 lines) |
| F | 0 = 5x7 dots | 1 = 5x10 dots |
| BF | 0 = Can accept instruction | 1 = Internal operation in progress |

1.3. Visible DDRAM addresses

#### 1.3.1. 1-line displays

Shown after reset (with N=0).

| Table1.5. DDRAM address usage for a 1-line LCD |  |  |
| --- | --- | --- |
| Display size | Visible |  |
|  | Character positions | DDRAM addresses |
| 1*8 | 00..07 | 00h..07h |
| 1*16 | 00..15 | 00h..0Fh |
| 1*20 | 00..19 | 00h..13h |
| 1*24 | 00..23 | 00h..17h |
| 1*32 | 00..31 | 00h..1Fh |
| 1*40 | 00..39 | 00h..27h |

#### 1.3.2. 2-line displays

Shown after reset (with N=1).

| Table1.6. DDRAM address usage for a 2-line LCD |  |  |
| --- | --- | --- |
| Display size | Visible |  |
|  | Character positions | DDRAM addresses |
| 2*16 | 00..15 | 00h..0Fh + 40h..4Fh |
| 2*20 | 00..19 | 00h..13h + 40h..53h |
| 2*24 | 00..23 | 00h..17h + 40h..57h |
| 2*32 | 00..31 | 00h..1Fh + 40h..5Fh |
| 2*40 | 00..39 | 00h..27h + 40h..67h |

1.3.3. 4-line displays

Shown after reset (with N=1).

| Table 1.7. DDRAM address usage for a 4-line LCD |  |  |
| --- | --- | --- |
| Display size | Visible |  |
|  | Character positions | DDRAM addresses |
| 4*16 | 00..15 | 00h..0Fh + 40h..4Fh + 14h..23h + 54h..63h |
| 4*20 | 00..19 | 00h..13h + 40h..53h + 14h..27h + 54h..67h |
| 4*40 | (00..39) on 1st controller and (00..39) on 2nd controller | (00h..27h + 40h..67h) on 1st controller and (00h..27h + 40h..67h) on 2nd controller |

### 1.4. Interfacing

#### 1.4.1. 8-bit interface

Example of busy flag testing using an 8-bit interface.

1.4.2. 4-bit interface

Example of busy flag testing using a 4-bit interface.

Example of data transfer using a 4-bit interface.

### 1.5. Characterset for 5x7 dot font