# PIC Example

# Code-examples for PIC16C84

# 1. Basic control software

Microchip's AN587 was used as a basis for this code.

WARNING:
Microchip's AN587 has major errors in the read from LCD code sequences.
The routines on this page use the correct read from LCD code sequences.

1.1. Requirements / features

- HD44780-based (industry-standard) character-LCD, all software in this chapter is based on it's instruction-set.
- PIC16C84 running on a 4MHz crystal, some code is based on this frequency.
- 8-bit interface between microcontroller and LCD-module.

1.2. Global declarations

#### 1.2.1. Register declarations

Purpose:

- Tells MPASM which ports and registers (files) to use.

Code:

LCD_DATA        EQU	PORTB           ; LCD data lines interface

LCD_DATA_TRIS   EQU	TRISB

LCD_CTRL        EQU	PORTA           ; LCD control lines interface

LCD_TEMP        EQU	0x020           ; LCD subroutines internal use

DELAY           EQU	0x023           ; Used in DELAYxxx routines

X_DELAY         EQU	0x024           ; Used in X_DELAYxxx routines

#### 1.2.2. Literal declarations

Purpose:

- Literal declarations (Equates) used in the code.

Code:

; PORTA control bits

LCD_E           EQU	2               ; LCD Enable control line

LCD_RW          EQU	1               ; LCD Read/Write control line

LCD_RS          EQU	0               ; LCD Register-Select control line

#### 1.2.3. Procedure declarations / library interface

Since MPLIB and MPLINK are not yet available, no declarations are needed.

1.3. Code

#### 1.3.1. LCD initialisation

Purpose:

- LCD initialisiation code to be executed after power-up (i.e.: before any other subroutines are used).

- Should be modified to your needs (i.e. display type, cursor on/off, etc.)

Code:

LCDINIT

; Busy-flag is not yet valid

CLRF        LCD_CTRL            ; ALL PORT output should output Low.

; power-up delay

MOVLW       0x01E

CALL        X_DELAY500          ; 30 * 0.5mS = 15mS

; Busy Flag should be valid from here

MOVLW       0x038               ; 8-bit-interface, 2-lines

CALL        LCDPUTCMD

MOVLW       0x000               ; disp.off, curs.off, no-blink

CALL        LCDDMODE

CALL        LCDCLEAR

MOVLW       0x004               ; disp.on, curs.off

CALL        LCDDMODE

MOVLW       0x002               ; auto-inc (shift-cursor)

CALL        LCDEMODE

RETURN

#### 1.3.2. Busy flag

Purpose:

- Tests if the LCD is busy. Returns when LCD busy-flag is inactive.

Code:

LCDBUSY

BSF         STATUS,RP0          ; Select Register page 1

MOVLW       0x0FF               ; Set PORTB for input

MOVWF       LCD_DATA_TRIS

BCF         STATUS, RP0         ; Select Register page 0

BCF         LCD_CTRL, LCD_RS    ; Set LCD for command mode

BSF         LCD_CTRL, LCD_RW    ; Setup to read busy flag

BSF         LCD_CTRL, LCD_E     ; LCD E-line High

MOVF        LCD_DATA, W         ; Read busy flag + DDram address

BCF         LCD_CTRL, LCD_E     ; LCD E-line Low

ANDLW       0x80                ; Check Busy flag, High = Busy

BTFSS       STATUS, Z

GOTO        LCDBUSY

LCDNOTBUSY

BCF         LCD_CTRL, LCD_RW

BSF         STATUS, RP0         ; Select Register page 1

MOVLW       0x000

MOVWF       LCD_DATA_TRIS       ; Set PORTB for output

BCF         STATUS, RP0         ; Select Register page 0

RETURN

#### 1.3.3. Clear display

Purpose:

- Clears display and returns cursor to home position (upper-left corner).

Code:

LCDCLEAR

MOVLW       0x001

CALL        LCDPUTCMD

RETURN

#### 1.3.4. Cursor home

Purpose:

- Returns cursor to home position.

- Returns display to original position (when shifted).

Code:

LCDHOME

MOVLW       0x002

CALL        LCDPUTCMD

RETURN

1.3.5. Entry mode

Purpose:

- Sets entry mode of the LCD

- Required entry mode must be set in W

b0    : 0 = no display shift, 1 = display shift

b1    : 0 = auto-decrement, 1 = auto-increment

b2-b7 : don't care

Code:

LCDEMODE

ANDLW       0x003               ; Strip upper bits

IORLW       0x004               ; Function set

CALL        LCDPUTCMD

RETURN

#### 1.3.6. Display mode

Purpose:

- Sets display control

- Required entry mode must be set in W

b0    : 0 = cursor blink off, 1 = cursor blink on (if b1 = 1)

b1    : 0 = cursor off,  1 = cursor on

b2    : 0 = display off, 1 = display on (display data remains in DD-RAM)

b3-b7 : don't care

Code:

LCDDMODE

ANDLW       0x007               ; Strip upper bits

IORLW       0x008               ; Function set

CALL        LCDPUTCMD

RETURN

1.3.7. Set character generator RAM address

Purpose:

- Sets the Character-Generator-RAM address. CGRAM data is read/written after this setting.

- Required CGRAM address must be set in W

b0-5	: required CGRAM address

b6-7	: don't care

Code:

LCDSCGA

ANDLW       0x03F               ; Strip upper bits

IORLW       0x040               ; Function set

CALL        LCDPUTCMD

RETURN

#### 1.3.8. Set display data RAM address

Purpose:

- Sets the Display-Data-RAM address. DDRAM data is read/written after this setting.

- Required entry mode must be set in W

b0-6	: required DDRAM address

b7	: don't care

Code:

LCDSDDA

IORLW       0x080               ; Function set

CALL        LCDPUTCMD

RETURN

1.3.9. Get address counter contents

Purpose:

- Returns address counter contents, used for both DDRAM and CGRAM.

- RAM address is returned in W

Code:

LCDGADDR

BSF         STATUS,RP0          ; Select Register page 1

MOVLW       0x0FF               ; Set PORTB for input

MOVWF       LCD_DATA_TRIS

BCF         STATUS, RP0         ; Select Register page 0

BCF         LCD_CTRL, LCD_RS    ; Set LCD for command mode

BSF         LCD_CTRL, LCD_RW    ; Setup to read busy flag

BSF         LCD_CTRL, LCD_E     ; LCD E-line High

MOVF        LCD_DATA, W         ; Read busy flag + RAM address

BCF         LCD_CTRL, LCD_E     ; LCD E-line Low

ANDLW       0x07F               ; Strip upper bit

BCF         LCD_CTRL, LCD_RW

BSF         STATUS, RP0         ; Select Register page 1

MOVLW       0x000

MOVWF       LCD_DATA_TRIS       ; Set PORTB for output

BCF         STATUS, RP0         ; Select Register page 0

RETURN

1.3.10. Write character

Purpose:

- Sends character to LCD

- Required character must be in W

Code:

LCDPUTCHAR

MOVWF       LCD_TEMP            ; Character to send is in W

CALL        LCDBUSY             ; Wait for LCD to be ready

BCF         LCD_CTRL, LCD_RW    ; Set LCD in read mode

BSF         LCD_CTRL, LCD_RS    ; Set LCD in data mode

BSF         LCD_CTRL, LCD_E     ; LCD E-line High

MOVF        LCD_TEMP, W

MOVWF       LCD_DATA            ; Send data to LCD

BCF         LCD_CTRL, LCD_E     ; LCD E-line Low

RETURN

#### 1.3.11. Write command

Purpose:

- Sends command to LCD

- Required command must be in W

Code:

LCDPUTCMD

MOVWF       LCD_TEMP            ; Command to send is in W

CALL        LCDBUSY             ; Wait for LCD to be ready

BCF         LCD_CTRL, LCD_RW    ; Set LCD in read mode

BCF         LCD_CTRL, LCD_RS    ; Set LCD in command mode

BSF         LCD_CTRL, LCD_E     ; LCD E-line High

MOVF        LCD_TEMP, W

MOVWF       LCD_DATA            ; Send data to LCD

BCF         LCD_CTRL, LCD_E     ; LCD E-line Low

RETURN

#### 1.3.12. Delay loops

Purpose:

- Used in LCDINIT subroutine

- Required delay factor must be in W

(Could be coded more efficient, but this approach gives more flexibility)

Code:

;***********************************  a 500uS delay @ 4MHz X-tal

DELAY500

MOVLW       D'165'              ; +1        1 cycle

MOVWF       DELAY               ; +2        1 cycle

DELAY500_LOOP

DECFSZ      DELAY, F            ; step1     1 cycle

GOTO        DELAY500_LOOP       ; step2     2 cycles

DELAY500_END

RETURN                          ; +3        2 cycles

;***********************************  a delay of 'W' * 500mS

X_DELAY500

MOVWF       X_DELAY             ; +1        1 cycle

X_DELAY500_LOOP

CALL        DELAY500            ; step1     wait 500uSec

DECFSZ      X_DELAY, F          ; step2     1 cycle

GOTO        X_DELAY500_LOOP     ; step3     2 cycles

X_DELAY500_END

RETURN                          ; +2        2 cycles

### 2. Advanced control software

### 2.1. User defined characters

Purpose:

After several requests a quick explanation on how to implement

user-defined characters:

First you'll need to make a pixel definition for the characters

you want to use.  This is the pixel definition for an underlined

'0' (char code 0x30) based on a 5x7 dots character definition:

|   bits    | byte

row  | 76543210  | value

------------------------

000  |     xxx   | 0x0E

001  |    x   x  | 0x11

010  |    x  xx  | 0x13

011  |    x x x  | 0x15

100  |    xx  x  | 0x19

101  |    x   x  | 0x11

110  |     xxx   | 0x0E

111  |    xxxxx  | 0x1F

The byte values need to be loaded into CGRAM address 00cccrrr

(binary), where:

-  ccc = user-defined character number (0...7)

-  rrr = row number of the user defined character (0...7)

Once that's done you can write character codes 0...7 to the

desired LCD character position, just like you do with

'normal' characters.

User-defined character definitions may be changed 'on-the-fly'.

While defining a 5x7 dots character:

- Character code bits (DDRAM) 2..0 correspond to CGRAM address bits 5..3

(i.e. 8 possible user defined characters).

While defining a 5x10 dots character:

- Character code bits (DDRAM) 2..1 correspond to CGRAM address bits 5..4

(i.e. 4 possible user defined characters).

It's best to switch off the cursor while writing to CGRAM.

Code:

(More detailed code may be published some day)

#### 3. Availability

LCD-PIC.ZIP: an example using some of the above subroutines (all subroutines are included).
Source is coded for a 4*20 LCD, adjust it to your needs!

Shows the following screen on a 4*20 LCD:

----------------------

|This is on line :  0|

|This is on line :  1|

|This is on line :  2|

|This is on line :  3|

----------------------

Shows the following screen on a 2*40 LCD:

-----------------------------------------

|This is on line :  0This is on line :  2|

|This is on line :  1This is on line :  3|

------------------------------------------

Shows the following screen on a 2*20 LCD:

----------------------

|This is on line :  0|

|This is on line :  1|

----------------------

### 4. Used hardware

#### 4.1 Controller

- A PIC16C84 is used to control the LCD.
- 8-bit data interface between controller and LCD.

4.2 Interface

### 5. Development environment

#### 5.1. Software

- Assembler: MPASM V1.30
- Programmer software: PICSTART 16B1 V5.00.00

#### 5.2. Hardware

- Programmer PICSTART 16B1 (firmware V2.00)